# INTELLIGENT WIRELESS SENSOR DATA COMMUNICATION SYSTEM
WITH ADAPTIVE MODULATION AND CHANNEL ESTIMATION

Simulation Platform: ESP32 + Wokwi
Programming Environment: MicroPython

# 1. Project Overview

This project implements an intelligent sensor-data communication chain on an ESP32 using Wokwi. The system acquires temperature data from a DS18B20 sensor and uses a potentiometer as a controllable channel-quality/SNR input. A packet is formed, protected with CRC, and passed through a lightweight K-nearest neighbor (KNN) classifier that selects BPSK or QPSK in automatic mode. Pilot symbols are inserted before a software-simulated wireless channel. The channel applies a complex channel coefficient and additive white Gaussian noise (AWGN). At the receiver, pilots are used to estimate the channel, the received symbols are equalized, demodulated, and evaluated using BER and CRC. Results are displayed on OLEDs and recorded by the performance and experiment modules.

# 2. Important Scope Statement

This is a Wokwi/ESP32 software simulation of a digital wireless communication chain. It does not claim that the two displayed nodes are exchanging real RF packets through ESP32 Wi-Fi. The wireless channel, fading, noise, modulation, channel estimation, equalization, BER calculation, and CRC verification are implemented in software to demonstrate communication-engineering principles.

# 3. Files Used in the Project

| File | Purpose | Required |
| --- | --- | --- |
| diagram.json | Wokwi circuit/component and wiring description. | Yes |
| main.py | Main ESP32 application and complete communication chain. | Yes |
| ml_model.py | Lightweight KNN classifier for BPSK/QPSK selection. | Yes |
| security.py | CRC frame generation, verification and payload extraction. | Yes |
| performance.py | BER/SNR/bit-error performance statistics. | Yes |
| experiment.py | Experiment-result logging and summary. | Yes |
| ssd1306.py | OLED driver used by MicroPython. | Yes |
| onewire.py | 1-Wire support for DS18B20. | Yes |
| ds18x20.py | DS18B20 temperature-sensor driver. | Yes |

# 4. Hardware / Wokwi Components

ESP32 development board.

DS18B20 temperature sensor.

Potentiometer used as a controllable channel/SNR input.

Two I2C OLED displays: TX and RX user-interface displays.

SEND push button.

MODE push button.

TX LED with series resistor.

RX LED with series resistor.

Breadboard/wiring and power/ground connections as represented in the final Wokwi diagram.

# 5. Main Pin Assignment

| Function | ESP32 GPIO | Description |
| --- | --- | --- |
| DS18B20 data | GPIO 4 | 1-Wire temperature input |
| Potentiometer | GPIO 34 | ADC channel/SNR control |
| SEND button | GPIO 18 | Active-low transmission trigger |
| MODE button | GPIO 19 | Active-low mode selection |
| TX LED | GPIO 2 | Transmission indication |
| RX LED | GPIO 5 | Reception indication |
| I2C SDA | GPIO 21 | OLED I2C data |
| I2C SCL | GPIO 22 | OLED I2C clock |
| TX OLED | 0x3C | I2C address |
| RX OLED | 0x3D | I2C address |

# 6. Operating Modes

Mode 0 — ML AUTO: KNN selects BPSK or QPSK from SNR, channel amplitude and estimated BER.

Mode 1 — BPSK: manual BPSK operation.

Mode 2 — QPSK: manual QPSK operation.

SEND button: executes one complete transmission and reception simulation.

MODE button: cycles through ML AUTO → BPSK → QPSK → ML AUTO.

# 7. Communication Procedure

1. Read temperature from DS18B20.

2. Read the potentiometer ADC value and map it to a simulated SNR range.

3. Convert the channel condition into features used by the KNN classifier.

4. Select BPSK or QPSK according to the selected operating mode.

5. Create a payload containing temperature and sequence number.

6. Append a CRC-32 protection field to the payload.

7. Convert the protected frame from bytes to bits.

8. Generate pilot symbols.

9. Modulate the information bits using BPSK or QPSK.

10. Combine pilots and data symbols into the transmit frame.

11. Pass the frame through the simulated complex channel and AWGN.

12. Separate pilot symbols from data symbols at the receiver.

13. Estimate the complex channel coefficient from the pilots.

14. Estimate channel SNR from pilot error power.

15. Equalize the received data using the estimated channel coefficient.

16. Demodulate the equalized data back into bits.

17. Calculate BER by comparing transmitted and received bits.

18. Reconstruct the received bytes.

19. Verify the CRC.

20. Extract and decode the payload if possible.

21. Record performance and experiment results.

22. Update TX/RX OLEDs and LED indicators.

# 8. Important Communication Equations

BPSK mapping: s = +1 for bit 0; s = −1 for bit 1.

QPSK mapping: s = (I + jQ)/√2, with I,Q ∈ {+1,−1}.

AWGN noise standard deviation: σ = √(1/(2·SNR_linear)), where SNR_linear = 10^(SNR_dB/10).

Received symbol model: y = h·x + n, where h is the complex channel coefficient and n is complex Gaussian noise.

Equalization: x̂ = y / ĥ.

BER: BER = N_errors / N_total_bits.

KNN Euclidean distance: d(x,y) = √Σᵢ(xᵢ−yᵢ)².

# 9. Complete Program Files

The following files form a self-contained version of the software architecture used by the project. No laptop IP address, MAC address, personal address, account information, or other personal data is required or included.

## 9.1 main.py

from machine import Pin, I2C, ADC
import time
import math
import random
import onewire
import ds18x20
import ssd1306
import ml_model
import performance
import security
import experiment

TEMP_PIN = 4
POT_PIN = 34
SEND_PIN = 18
MODE_PIN = 19
TX_LED_PIN = 2
RX_LED_PIN = 5
SDA_PIN = 21
SCL_PIN = 22

TX_ADDRESS = 0x3C
RX_ADDRESS = 0x3D

i2c = I2C(0, scl=Pin(SCL_PIN), sda=Pin(SDA_PIN), freq=100000)

oled_tx = None
oled_rx = None
devices = i2c.scan()

if TX_ADDRESS in devices:
    try:
        oled_tx = ssd1306.SSD1306_I2C(128, 64, i2c, addr=TX_ADDRESS)
    except Exception as e:
        print("TX OLED ERROR:", e)

if RX_ADDRESS in devices:
    try:
        oled_rx = ssd1306.SSD1306_I2C(128, 64, i2c, addr=RX_ADDRESS)
    except Exception as e:
        print("RX OLED ERROR:", e)

onewire_bus = onewire.OneWire(Pin(TEMP_PIN))
ds_sensor = ds18x20.DS18X20(onewire_bus)
roms = ds_sensor.scan()

pot = ADC(Pin(POT_PIN))
try:
    pot.atten(ADC.ATTN_11DB)
except:
    pass

send_button = Pin(SEND_PIN, Pin.IN, Pin.PULL_UP)
mode_button = Pin(MODE_PIN, Pin.IN, Pin.PULL_UP)

tx_led = Pin(TX_LED_PIN, Pin.OUT)
rx_led = Pin(RX_LED_PIN, Pin.OUT)
tx_led.off()
rx_led.off()

sequence = 0
mode = 0
previous_send = send_button.value()
previous_mode = mode_button.value()
last_send_time = time.ticks_ms()
last_mode_time = time.ticks_ms()
DEBOUNCE = 250

random.seed(time.ticks_ms())
performance.start()
experiment.reset()

def tx_screen(lines):
    if oled_tx is None:
        return
    oled_tx.fill(0)
    y = 0
    for text in lines:
        oled_tx.text(str(text), 0, y)
        y += 10
        if y >= 64:
            break
    oled_tx.show()

def rx_screen(lines):
    if oled_rx is None:
        return
    oled_rx.fill(0)
    y = 0
    for text in lines:
        oled_rx.text(str(text), 0, y)
        y += 10
        if y >= 64:
            break
    oled_rx.show()

def both_screen(tx_lines, rx_lines):
    tx_screen(tx_lines)
    rx_screen(rx_lines)

def read_temperature():
    if len(roms) == 0:
        return None
    try:
        ds_sensor.convert_temp()
        time.sleep_ms(750)
        return ds_sensor.read_temp(roms[0])
    except Exception as e:
        print("TEMPERATURE ERROR:", e)
        return None

def read_channel():
    raw = pot.read()
    ratio = raw / 4095.0
    snr = 2.0 + ratio * 18.0
    return raw, snr

def estimate_channel_amplitude(snr):
    quality = (snr - 2.0) / 18.0
    quality = max(0.0, min(1.0, quality))
    return 0.55 + 0.45 * quality

def modulation_name(snr, channel_amplitude, ber_estimate):
    if mode == 1:
        return "BPSK", 100.0
    if mode == 2:
        return "QPSK", 100.0
    features = [snr, channel_amplitude, ber_estimate]
    return (ml_model.predict(features),
            ml_model.confidence(features) * 100.0)

def bytes_to_bits(data):
    bits = []
    for byte in data:
        for bit_position in range(7, -1, -1):
            bits.append((byte >> bit_position) & 1)
    return bits

def bits_to_bytes(bits):
    result = bytearray()
    for start in range(0, len(bits), 8):
        value = 0
        for j in range(8):
            value <<= 1
            index = start + j
            if index < len(bits):
                value |= bits[index]
        result.append(value)
    return bytes(result)

def bpsk_modulate(bits):
    return [complex(1, 0) if bit == 0 else complex(-1, 0) for bit in bits]

def bpsk_demodulate(symbols):
    return [0 if s.real >= 0 else 1 for s in symbols]

def qpsk_modulate(bits):
    work = list(bits)
    if len(work) % 2:
        work.append(0)
    symbols = []
    for i in range(0, len(work), 2):
        b1, b2 = work[i], work[i + 1]
        if b1 == 0 and b2 == 0:
            real, imag = 1, 1
        elif b1 == 0 and b2 == 1:
            real, imag = 1, -1
        elif b1 == 1 and b2 == 0:
            real, imag = -1, 1
        else:
            real, imag = -1, -1
        symbols.append(complex(real / math.sqrt(2), imag / math.sqrt(2)))
    return symbols

def qpsk_demodulate(symbols):
    bits = []
    for s in symbols:
        bits.append(0 if s.real >= 0 else 1)
        bits.append(0 if s.imag >= 0 else 1)
    return bits

def gaussian():
    u1 = max(random.random(), 0.000001)
    u2 = random.random()
    return math.sqrt(-2.0 * math.log(u1)) * math.cos(2.0 * math.pi * u2)

def db_to_linear(db):
    return 10 ** (db / 10.0)

def linear_to_db(value):
    if value <= 0:
        return -60.0
    return 10.0 * math.log10(value)

def make_pilots():
    return [complex(1, 0) for _ in range(16)]

def channel(symbols, snr_db):
    snr_linear = db_to_linear(snr_db)
    quality = max(0.0, min(1.0, (snr_db - 2.0) / 18.0))
    amplitude = 0.55 + 0.45 * quality
    phase = random.uniform(-0.35, 0.35)
    h = complex(amplitude * math.cos(phase),
                amplitude * math.sin(phase))
    sigma = math.sqrt(1.0 / (2.0 * snr_linear))
    received = []
    for symbol in symbols:
        noise = complex(gaussian() * sigma, gaussian() * sigma)
        received.append(h * symbol + noise)
    return received, h

def estimate_channel(received_pilots, transmitted_pilots):
    count = min(len(received_pilots), len(transmitted_pilots))
    if count == 0:
        return complex(1, 0)
    h = complex(0, 0)
    for i in range(count):
        if abs(transmitted_pilots[i]) > 0.000001:
            h += received_pilots[i] / transmitted_pilots[i]
    return h / count

def equalize(symbols, channel_estimate):
    if abs(channel_estimate) < 0.000001:
        channel_estimate = complex(1, 0)
    return [s / channel_estimate for s in symbols]

def estimate_snr(received, transmitted, h):
    count = min(len(received), len(transmitted))
    if count == 0:
        return 0.0
    signal_power = 0.0
    error_power = 0.0
    for i in range(count):
        expected = h * transmitted[i]
        signal_power += abs(expected) ** 2
        error_power += abs(received[i] - expected) ** 2
    signal_power /= count
    error_power /= count
    if error_power < 0.0000001:
        return 30.0
    return linear_to_db(signal_power / error_power)

def calculate_ber(transmitted, received):
    count = min(len(transmitted), len(received))
    if count == 0:
        return 1.0, 0, 0
    errors = sum(1 for i in range(count) if transmitted[i] != received[i])
    return errors / float(count), errors, count

def make_payload(temperature, seq):
    if temperature is None:
        temperature = 0.0
    return ("T={:.2f};S={:03d}".format(temperature, seq)).encode()

def transmit():
    global sequence
    sequence += 1

    temperature = read_temperature()
    adc_value, snr = read_channel()
    channel_amplitude = estimate_channel_amplitude(snr)

    # This is an initial operating estimate. A later experiment can replace
    # it with a measured/history-based estimate.
    initial_ber_estimate = 0.02
    modulation, ml_confidence = modulation_name(
        snr, channel_amplitude, initial_ber_estimate
    )

    payload = make_payload(temperature, sequence)
    frame = security.append_crc(payload)
    bits = bytes_to_bits(frame)

    tx_led.on()
    rx_led.off()

    tx_screen([
        "TX NODE",
        "T:{:.1f}C".format(temperature if temperature is not None else 0),
        "SNR:{:.1f}dB".format(snr),
        "ML:" + modulation,
        "SEQ:{:03d}".format(sequence),
        "CRC:" + security.crc_string(frame)
    ])

    rx_screen([
        "RX NODE",
        "RECEIVING...",
        "",
        "MODE:" + modulation,
        "SEQ:{:03d}".format(sequence)
    ])

    time.sleep_ms(250)

    pilots = make_pilots()
    data_symbols = bpsk_modulate(bits) if modulation == "BPSK" else qpsk_modulate(bits)
    tx_symbols = pilots + data_symbols

    rx_symbols, actual_h = channel(tx_symbols, snr)
    pilot_count = len(pilots)
    rx_pilots = rx_symbols[:pilot_count]
    rx_data = rx_symbols[pilot_count:]

    estimated_h = estimate_channel(rx_pilots, pilots)
    measured_snr = estimate_snr(rx_pilots, pilots, estimated_h)
    equalized = equalize(rx_data, estimated_h)

    if modulation == "BPSK":
        rx_bits = bpsk_demodulate(equalized)
    else:
        rx_bits = qpsk_demodulate(equalized)

    rx_bits = rx_bits[:len(bits)]
    ber, errors, total = calculate_ber(bits, rx_bits)

    recovered_frame = bits_to_bytes(rx_bits)
    crc_ok = security.verify_crc(recovered_frame)
    recovered_payload = security.extract_payload(recovered_frame)

    try:
        recovered_text = recovered_payload.decode()
    except:
        recovered_text = "DECODE ERROR"

    performance.record(total, errors, measured_snr, ber, crc_ok)
    experiment.add_result(
        sequence, modulation, measured_snr, ber, crc_ok, errors, total
    )

    crc_status = "CRC:VALID" if crc_ok else "CRC:INVALID"

    tx_led.off()
    rx_led.on()

    rx_screen([
        "RX NODE",
        "T:{:.1f}C".format(temperature if temperature is not None else 0),
        "ML:" + modulation,
        "BER:{:.3f}".format(ber),
        crc_status,
        "SEQ:{:03d}".format(sequence)
    ])

    print("\n========================================")
    print("TRANSMISSION #", sequence)
    print("Temperature:", temperature)
    print("ADC:", adc_value)
    print("Input SNR:", round(snr, 2), "dB")
    print("Channel amplitude:", round(channel_amplitude, 3))
    print("Modulation:", modulation)
    print("ML confidence:", round(ml_confidence, 1), "%")
    print("Payload:", payload.decode())
    print("CRC:", security.crc_string(frame))
    print("TX bits:", len(bits))
    print("Actual H:", actual_h)
    print("Estimated H:", estimated_h)
    print("Measured SNR:", round(measured_snr, 2), "dB")
    print("Bit errors:", errors, "/", total)
    print("BER:", "{:.6f}".format(ber))
    print("CRC:", "VALID" if crc_ok else "INVALID")
    print("Recovered payload:", recovered_text)
    print("Total experiments:", experiment.count())
    performance.print_summary()
    experiment.print_summary()

    time.sleep_ms(700)
    rx_led.off()

def change_mode():
    global mode
    mode += 1
    if mode > 2:
        mode = 0
    name = "ML AUTO" if mode == 0 else ("BPSK" if mode == 1 else "QPSK")
    print("MODE CHANGED:", name)
    both_screen(["TX NODE", "MODE CHANGE", "", name],
                ["RX NODE", "MODE CHANGE", "", name])
    time.sleep_ms(700)
    tx_screen(["TX NODE", "READY", "", "MODE:" + name, "", "PRESS SEND"])
    rx_screen(["RX NODE", "READY", "", "MODE:" + name, "", "WAITING..."])

print("========================================")
print("COMMUNICATION SYSTEM")
print("INTELLIGENT WIRELESS SENSOR DATA")
print("COMMUNICATION SYSTEM")
print("========================================")
print("ML MODEL     : ENABLED")
print("BPSK         : ENABLED")
print("QPSK         : ENABLED")
print("PILOTS       : ENABLED")
print("CHANNEL EST. : ENABLED")
print("EQUALIZER    : ENABLED")
print("BER          : ENABLED")
print("CRC          : ENABLED")
print("EXPERIMENT   : ENABLED")
print("MODE 0: ML AUTO | MODE 1: BPSK | MODE 2: QPSK")
print("PRESS SEND TO TRANSMIT")

both_screen(
    ["PROJECT", "WIRELESS TX", "", "ML+CRC READY", "", "PRESS SEND"],
    ["PROJECT", "WIRELESS RX", "", "EXP READY", "", "WAITING..."]
)

while True:
    current_send = send_button.value()
    if previous_send == 1 and current_send == 0:
        now = time.ticks_ms()
        if time.ticks_diff(now, last_send_time) >= DEBOUNCE:
            transmit()
            last_send_time = now
    previous_send = current_send

    current_mode = mode_button.value()
    if previous_mode == 1 and current_mode == 0:
        now = time.ticks_ms()
        if time.ticks_diff(now, last_mode_time) >= DEBOUNCE:
            change_mode()
            last_mode_time = now
    previous_mode = current_mode
    time.sleep_ms(20)

## 9.2 ml_model.py

# Lightweight KNN modulation classifier for ESP32 MicroPython.
# 0 = BPSK, 1 = QPSK

TRAINING_DATA = [
    [2.0,0.55,0.30],[3.0,0.57,0.25],[4.0,0.59,0.20],
    [5.0,0.61,0.16],[6.0,0.63,0.12],[7.0,0.65,0.09],
    [8.0,0.67,0.07],[8.5,0.68,0.06],[9.0,0.69,0.05],
    [9.5,0.70,0.045],[10.0,0.71,0.035],
    [10.2,0.72,0.032],[10.5,0.73,0.028],
    [10.8,0.74,0.024],[11.0,0.75,0.020],
    [11.2,0.76,0.018],[11.5,0.77,0.016],[12.0,0.78,0.013],
    [12.5,0.79,0.011],[13.0,0.80,0.009],[14.0,0.82,0.007],
    [15.0,0.84,0.005],[16.0,0.86,0.004],[17.0,0.88,0.003],
    [18.0,0.90,0.002],[19.0,0.92,0.0015],[20.0,0.94,0.001]
]

TRAINING_LABELS = [0]*15 + [1]*13

def normalize_features(features):
    snr, channel, ber = features
    snr_n = max(0.0, min(1.0, snr / 20.0))
    channel_n = max(0.0, min(1.0, (channel - 0.5) / 0.5))
    ber_n = max(0.0, min(1.0, ber / 0.30))
    return [snr_n, channel_n, ber_n]

NORMALIZED_DATA = [normalize_features(x) for x in TRAINING_DATA]

def distance(a, b):
    total = 0.0
    for i in range(len(a)):
        d = a[i] - b[i]
        total += d * d
    return total ** 0.5

def _votes(features, k=5):
    sample = normalize_features(features)
    distances = []
    for i in range(len(NORMALIZED_DATA)):
        distances.append((distance(sample, NORMALIZED_DATA[i]),
                          TRAINING_LABELS[i]))
    distances.sort(key=lambda x: x[0])
    nearest = distances[:min(k, len(distances))]
    bpsk_votes = 0
    qpsk_votes = 0
    for _, label in nearest:
        if label == 0:
            bpsk_votes += 1
        else:
            qpsk_votes += 1
    return bpsk_votes, qpsk_votes, len(nearest)

def predict(features, k=5):
    bpsk, qpsk, _ = _votes(features, k)
    return "QPSK" if qpsk > bpsk else "BPSK"

def confidence(features, k=5):
    bpsk, qpsk, n = _votes(features, k)
    if n == 0:
        return 0.0
    return max(bpsk, qpsk) / float(n)

def model_info():
    return {
        "algorithm": "KNN",
        "features": ["SNR", "Channel Amplitude", "BER"],
        "classes": ["BPSK", "QPSK"],
        "samples": len(TRAINING_DATA)
    }

def test_model():
    samples = [
        [5.0,0.62,0.15],
        [9.0,0.69,0.05],
        [12.0,0.78,0.013],
        [16.0,0.86,0.004],
        [19.0,0.92,0.0015]
    ]
    print("========== ML MODEL TEST ==========")
    for s in samples:
        print(s, "->", predict(s),
              "confidence:", round(confidence(s)*100,1), "%")
    print("====================================")

## 9.3 security.py

# CRC-32 frame protection. No network address or personal data is used.

POLY = 0xEDB88320
MASK = 0xFFFFFFFF

def crc32(data):
    crc = MASK
    for byte in data:
        crc ^= byte
        for _ in range(8):
            if crc & 1:
                crc = (crc >> 1) ^ POLY
            else:
                crc >>= 1
    return crc ^ MASK

def append_crc(payload):
    value = crc32(payload)
    return payload + value.to_bytes(4, "big")

def verify_crc(frame):
    if len(frame) < 4:
        return False
    payload = frame[:-4]
    received = int.from_bytes(frame[-4:], "big")
    return crc32(payload) == received

def extract_payload(frame):
    if len(frame) < 4:
        return b""
    return frame[:-4]

def crc_string(frame):
    if len(frame) < 4:
        return "00000000"
    value = int.from_bytes(frame[-4:], "big")
    return "{:08X}".format(value)

## 9.4 performance.py

# Runtime communication-performance statistics.

_total_bits = 0
_total_errors = 0
_runs = 0
_ber_sum = 0.0
_snr_sum = 0.0
_crc_ok = 0

def start():
    global _total_bits, _total_errors, _runs, _ber_sum, _snr_sum, _crc_ok
    _total_bits = 0
    _total_errors = 0
    _runs = 0
    _ber_sum = 0.0
    _snr_sum = 0.0
    _crc_ok = 0

def record(total_bits, errors, snr, ber, crc_ok):
    global _total_bits, _total_errors, _runs, _ber_sum, _snr_sum, _crc_ok
    _total_bits += total_bits
    _total_errors += errors
    _runs += 1
    _ber_sum += ber
    _snr_sum += snr
    if crc_ok:
        _crc_ok += 1

def print_summary():
    if _runs == 0:
        print("PERFORMANCE: no experiments")
        return
    avg_ber = _ber_sum / _runs
    avg_snr = _snr_sum / _runs
    overall_ber = _total_errors / float(_total_bits) if _total_bits else 0.0
    print("========== PERFORMANCE SUMMARY ==========")
    print("Experiments:", _runs)
    print("Total bits:", _total_bits)
    print("Total errors:", _total_errors)
    print("Overall BER:", "{:.6f}".format(overall_ber))
    print("Average BER:", "{:.6f}".format(avg_ber))
    print("Average SNR:", "{:.2f} dB".format(avg_snr))
    print("CRC success:", "{}/{}".format(_crc_ok, _runs))
    print("=========================================")

## 9.5 experiment.py

# Simple in-memory experiment logger for ESP32 MicroPython.

_results = []

def reset():
    global _results
    _results = []

def add_result(sequence, modulation, snr, ber, crc_ok, errors, total):
    _results.append({
        "sequence": sequence,
        "modulation": modulation,
        "snr": snr,
        "ber": ber,
        "crc": crc_ok,
        "errors": errors,
        "total": total
    })

def count():
    return len(_results)

def print_summary():
    print("========== EXPERIMENT SUMMARY ==========")
    print("Runs:", len(_results))
    for r in _results:
        print(
            "SEQ", r["sequence"],
            "|", r["modulation"],
            "| SNR", "{:.2f}".format(r["snr"]),
            "| BER", "{:.6f}".format(r["ber"]),
            "| CRC", "VALID" if r["crc"] else "INVALID",
            "| ERR", r["errors"], "/", r["total"]
        )
    print("=========================================")

## 9.6 External / Support Files

The project also uses the standard MicroPython/Wokwi support drivers named ssd1306.py, onewire.py and ds18x20.py. These are library/driver files rather than project-specific communication algorithms. They should be added to the Wokwi project from the appropriate MicroPython/Wokwi library source. Their internal source is not reproduced here so that the report does not incorrectly claim authorship of third-party driver code.

# 10. Wokwi Setup Procedure

1. Create an ESP32 Wokwi project configured for MicroPython.

2. Place the ESP32, DS18B20, potentiometer, two SSD1306-compatible I2C OLED displays, two push buttons, two LEDs and their required resistors.

3. Wire the DS18B20 data line to GPIO 4 and provide appropriate power/ground.

4. Connect the potentiometer output to GPIO 34 and its end terminals to supply and ground.

5. Connect SEND to GPIO 18 and MODE to GPIO 19. The program uses internal pull-ups, so the button is active-low.

6. Connect the LEDs to GPIO 2 and GPIO 5 through suitable resistors.

7. Connect the OLED I2C bus to SDA GPIO 21 and SCL GPIO 22.

8. Use I2C address 0x3C for the TX display and 0x3D for the RX display.

9. Add main.py, ml_model.py, security.py, performance.py and experiment.py to the project.

10. Add the required SSD1306, OneWire and DS18B20 driver files.

11. Start the simulation and wait for the READY screens.

12. Press MODE to cycle through ML AUTO, BPSK and QPSK.

13. Press SEND to execute one complete communication experiment.

14. Rotate the potentiometer and press SEND again to observe how the simulated channel/SNR condition changes.

15. Observe BER, CRC, sequence number, modulation and SNR on the OLEDs and inspect the serial console when available.

# 11. Expected Operation / Output

TX OLED: temperature, SNR, selected modulation, sequence number and CRC value.

RX OLED: temperature, selected modulation, BER, CRC status and sequence number.

TX LED: active during the simulated transmission stage.

RX LED: active after the receiver-processing stage.

SEND increases the sequence number for each completed transmission.

Changing the potentiometer changes the simulated SNR/channel quality.

In ML AUTO, the KNN model chooses BPSK or QPSK from the configured training data and current features.

In manual modes, the MODE button directly selects BPSK or QPSK.

CRC: VALID indicates that the reconstructed protected frame passed the CRC check; CRC: INVALID indicates a detected frame error.

BER reports the fraction of compared bits that differ between the transmitted and recovered bit streams.

# 12. Interpretation of the Project

The project demonstrates the structure of a digital communication receiver/transmitter chain rather than only displaying sensor values. The sensor reading becomes a packet, the packet is protected, mapped to digital symbols, passed through a mathematical channel model, estimated and equalized at the receiver, demodulated, checked using BER and CRC, and finally displayed/logged.

# 13. Important Limitations

The wireless channel is mathematical/software-simulated; there is no real RF propagation measurement.

The KNN training data are manually constructed demonstration samples, not a large experimentally measured dataset.

The current ML decision uses an initial BER estimate rather than a continuously learned history model.

Only BPSK and QPSK are included.

The channel model is simplified and does not represent every real-world multipath, Doppler, synchronization or RF impairment.

The experiment logger stores results in RAM and is therefore not a permanent database.

The project should be described as a simulation/activity implementation unless and until it is validated with real hardware and measured wireless data.

# 14. Recommended Future Upgrades

Replace the manually generated KNN training data with measured or statistically generated channel datasets.

Use measured BER history as an ML feature rather than a fixed initial estimate.

Add QPSK Gray coding explicitly and compare theoretical and simulated BER curves.

Add configurable Rayleigh/Rician fading, multipath taps and Doppler.

Add synchronization and carrier/phase-offset models.

Run controlled SNR sweeps and generate BER-versus-SNR graphs.

Compare KNN against decision-tree, logistic-regression or small neural-network classifiers.

Store experiment results in CSV format for offline analysis.

Validate the algorithm with two actual ESP32 nodes and a real communication link before claiming hardware wireless performance.

# 15. Project Reproducibility Checklist

☐ ESP32 Wokwi project created.

☐ All hardware components placed.

☐ GPIO wiring matches the pin table.

☐ OLED addresses configured as 0x3C and 0x3D.

☐ main.py uploaded.

☐ ml_model.py uploaded.

☐ security.py uploaded.

☐ performance.py uploaded.

☐ experiment.py uploaded.

☐ SSD1306 / OneWire / DS18B20 support files available.

☐ Simulation starts without import errors.

☐ SEND button increments sequence number.

☐ MODE button cycles modes.

☐ Potentiometer changes simulated SNR.

☐ BER and CRC are displayed after transmission.

# 16. Privacy / Personal-Information Statement

This document intentionally contains no laptop IP address, MAC address, network address, physical address, account number, password, private key, personal contact information, or other private system information. The only personal field intentionally left available is the project-author name field on the cover page.

# 17. Final Project Statement

The implementation is an activity-stage research prototype. It is intended to demonstrate the end-to-end concept of intelligent modulation selection, pilot-assisted channel estimation, equalization, BER evaluation and CRC-based integrity checking in a controlled ESP32/Wokwi environment. Further validation, measurement, dataset development and hardware experimentation can be carried out as subsequent research work.